BACnet/IP Server

Conformance Statement

Brady Worldwide, Inc.

6555 W Good Hope Road Milwaukee, WI 53223

(p) 888-250-3082 (w) www.bradyid.com

Date: 11/18/2024

Vendor Name: Brady Corporation

Product Name: V1400

Product Model Number: 1

Application Software Version: 1 BACnet Protocol Revision: 24

Product Description:

The Brady V1400 Industrial Ethernet Bluetooth Cradle allows a BACnet/IP Controller to connect to the Cradle. The V1400 Cradle provides its data in BACnet via AI, AO, BI, BO, and CSV objects.

BACnet Standardized Device Profiles Supported (Annex L):

 BACnet Cross-Domain Advanced Operator Workstation (B-XAWS)

 BACnet Advanced Operator Workstation (B-AWS)

 BACnet Operator Workstation (B-OWS)

 BACnet Operator Display (B-OD)

 BACnet Advanced Life Safety Workstation (B-ALSWS)

 BACnet Life Safety Workstation (B-LSWS)

 BACnet Life Safety Annunciator Panel (B-LSAP)

 BACnet Advanced Access Control Workstation (B-AACWS)

 BACnet Access Control Workstation (B-ACWS)

 BACnet Access Control Security Display (B-ACSD)

 BACnet Building Controller (B-BC)

 BACnet Advanced Application Controller (B-AAC)

 BACnet Application Specific Controller (B-ASC)

 BACnet Smart Actuator (B-SA)

 BACnet Smart Sensor (B-SS)

 BACnet Advanced Life Safety Controller (B-ALSC)

 BACnet Life Safety Controller (B-LSC)

 BACnet Advanced Access Control Controller (B-AACC)

 BACnet Access Control Controller (B-ACC)

 BACnet Router (B-RTR)

 BACnet Gateway (B-GW)

 BACnet Broadcast Management Device (B-BBMD)

 BACnet Access Control Door Controller (B-ACDC)

 BACnet Access Control Credential Reader (B-ACCR)

 BACnet General (B-GENERAL)

BACnet Interoperability Building Blocks Supported (Annex K):

DS-RP-B Data Sharing ReadProperty - B

DS-RPM-B Data Sharing ReadPropertyMultiple - B

DS-WP-B Data Sharing WriteProperty – B

DS-WPM-B Data Sharing WritePropertyMultiple - B

DS-COV-B Data Sharing COV-B

Segmentation Capability:

No Segmentation supported.

Standard Object Types Supported:

| Object Type Supported | Can Be Created Dynamically | Can Be Deleted Dynamically |
| --- | --- | --- |
| Device | No | No |
| Analog Input | No | No |
| Analog Output | No | No |
| Binary Input | No | No |
| Binary Output | No | No |
| Character String Value | No | No |

No optional properties are supported.

Data Link Layer Options:

 ARCNET (ATA 878.1), 2.5 Mb. (Clause 8)

 ARCNET (ATA 878.1), EIA-485 (Clause 8), baud rate(s) ____________

 BACnet IP, (Annex J)

 BACnet IP, (Annex J), BACnet Broadcast Management Device (BBMD)

 BACnet IP, (Annex J), Network Address Translation (NAT Traversal)

 BACnet IPv6, (Annex U)

 BACnet IPv6, (Annex U), BACnet Broadcast Management Device (BBMD)

 BACnet/ZigBee (Annex O) __________

 Ethernet, ISO 8802-3 (Clause 7)

 LonTalk, ISO/IEC 14908.1 (Clause 11), medium: __________

 MS/TP master (Clause 9), baud rate(s):

 MS/TP slave (Clause 9), baud rate(s):

 Point-To-Point, EIA 232 (Clause 10), baud rate(s):

 Point-To-Point, modem, (Clause 10), baud rate(s):

 Other:

Device Address Binding:

Is static device binding supported? (This is currently necessary for two-way communication with MS/TP slaves and

certain other devices.)  Yes  No

Networking Options:

 Router, Clause 6 - List all routing configurations, e.g., ARCNET-Ethernet, Ethernet-MS/TP, etc.

 Annex H, BACnet Tunneling Router over IP

Character Sets Supported:

Indicating support for multiple character sets does not imply that they can all be supported simultaneously.

 ISO 10646 (UTF-8)  IBM/Microsoft DBCS  ISO 8859-1

 ISO 10646 (UCS-2)  ISO 10646 (UCS-4)  JIS X 0208

Gateway Options:

If this product is a communication gateway, describe the types of non-BACnet equipment/networks(s) that the gateway supports:

Network Security Options:

 Non-secure Device - is capable of operating without BACnet Network Security

 Secure Device - is capable of using BACnet Network Security (NS-SD BIBB)

 Multiple Application-Specific Keys

 Supports encryption (NS-ED BIBB)

 Key Server (NS-KS BIBB)